UNIFIED NAMESPACE

Design Specification

Enterprise C — Biopharmaceutical Operations ISA-95 + ISA-88 ¦ Batch & Continuous Process ¦ AnyLog Historian Integration

| Document | UNS-EC-001 Unified Namespace Design Specification |
| --- | --- |
| Version | 1.0 (Draft — pending AnyLog data validation) |
| Standards | ISA-95 Levels 0-4 ¦ ISA-88 Batch Control |
| Scope | Bioreactor, TFF, Formulation, Fill/Finish, Utilities |
| Historian | AnyLog Distributed Edge Historian (anylog-proveit) |

# 1.  Purpose & Scope

This document defines the Unified Namespace (UNS) for Enterprise C's biopharmaceutical manufacturing site. The UNS provides a single, canonical source of truth for all operational data — from raw sensor readings at ISA-95 Level 0 through to enterprise KPIs at Level 4 — structured to support both ISA-95 equipment hierarchy and ISA-88 batch/phase control.

The namespace is implemented as the topic backbone for MQTT/SparkplugB message routing and as the logical hierarchy within the AnyLog distributed historian. All data published to or queried from AnyLog maps 1-to-1 to a UNS path.

## 1.1  Design Principles

- Single source of truth — every data point has exactly one canonical path

- Contextual structure — path encodes enterprise, site, area, unit, phase, and tag

- ISA-95 and ISA-88 aligned — hierarchy mirrors equipment and procedural models

- Immutable path prefix — namespace changes require formal change control

- Human-readable — paths are self-documenting without a separate data dictionary

# 2.  Namespace Architecture

## 2.1  Top-Level Structure

The UNS root follows a two-branch design separating raw process data from batch-contextualised data:

EnterpriseC/                         ← ISA-95 L4 Enterprise root

{Site}/                             ← ISA-95 L3 Site

process/                          ← Continuous / real-time process data

{Area}/                         ← ISA-95 L2 Area

{Unit}/                       ← ISA-95 L1 Work Centre

{Tag}                       ← L0 sensor / actuator reading

batch/                            ← ISA-88 batch-contextualised data

{Area}/

{Unit}/

{BatchID}/                  ← ISA-88 batch identifier

{Phase}/                  ← ISA-88 operation / phase

{Tag}

events/                           ← Alarms, state changes, audit

meta/                             ← Equipment metadata & recipes

## 2.2  ISA-95 Level Mapping

| Level | ISA-95 Term | Enterprise C Mapping |
| --- | --- | --- |
| L4 | Enterprise | EnterpriseC/ |
| L3 | Site | EnterpriseC/SiteA/ (primary manufacturing site) |
| L2 | Area | bioprocess/ ¦ tff/ ¦ formulation/ ¦ fillfin/ ¦ utilities/ |
| L1 | Work Centre | Unit250/ Unit251/ Unit260/ etc. |
| L0 | Process Cell | Individual tag readings (temperature, pH, DO, etc.) |

# 3.  Process Areas & Units

## 3.1  Bioprocess Area  (Upstream)

Covers seed train, N-1, and production bioreactors. The primary production unit is Unit 250, a fed-batch stirred tank bioreactor.

| UNS Path Segment | Unit ID | Description |
| --- | --- | --- |
| bioprocess/Unit250 | Unit250 | Production bioreactor — fed-batch, 2000L nominal |
| bioprocess/Unit251 | Unit251 | N-1 seed bioreactor |
| bioprocess/Unit252 | Unit252 | Seed train vessel 2 |
| bioprocess/Media | MediaPrep | Media preparation and hold tanks |

### Unit 250 — Tag Namespace

The following tag categories are defined under each bioreactor unit:

| Full UNS Path (example) | Tag Category | AnyLog Table / Column |
| --- | --- | --- |
| …/Unit250/temperature | Process | bioreactor_data.temperature_pv |
| …/Unit250/ph | Process | bioreactor_data.ph_pv |
| …/Unit250/dissolved_o2 | Process | bioreactor_data.do_pv |
| …/Unit250/agitation | Process | bioreactor_data.agitation_rpm |
| …/Unit250/co2_pct | Process | bioreactor_data.co2_percent |
| …/Unit250/feed_rate | Feed | bioreactor_data.feed_rate_pv |
| …/Unit250/base_rate | Feed | bioreactor_data.base_rate_pv |
| …/Unit250/air_flow | Gas | bioreactor_data.air_flow_pv |
| …/Unit250/o2_flow | Gas | bioreactor_data.o2_flow_pv |
| …/Unit250/equipment_state | State | bioreactor_data.equipment_state |
| …/Unit250/batch_id | Batch | bioreactor_data.batch_id |

## 3.2  TFF Area  (Downstream)

Tangential flow filtration for harvest clarification and concentration/diafiltration steps.

| UNS Path Segment | Unit ID | Description |
| --- | --- | --- |
| tff/TFF260 | TFF260 | Primary TFF — harvest clarification |
| tff/TFF261 | TFF261 | Secondary TFF — concentration / diafiltration |
| tff/TFF262 | TFF262 | Polishing TFF |

### TFF Tag Namespace

| Full UNS Path (example) | Tag Category | AnyLog Table / Column |
| --- | --- | --- |
| …/TFF260/feed_pressure | Pressure | tff_data.feed_pressure_pv |
| …/TFF260/permeate_pressure | Pressure | tff_data.permeate_pressure_pv |
| …/TFF260/retentate_pressure | Pressure | tff_data.retentate_pressure_pv |
| …/TFF260/flux | Flow | tff_data.flux_pv |
| …/TFF260/tmp | Derived | tff_data.tmp_calculated |
| …/TFF260/conductivity | Quality | tff_data.conductivity_pv |
| …/TFF260/batch_id | Batch | tff_data.batch_id |

## 3.3  Formulation  &  Fill/Finish

| UNS Path Segment | Unit ID | Description |
| --- | --- | --- |
| formulation/FormTank01 | FormTank01 | Bulk formulation vessel |
| formulation/FormTank02 | FormTank02 | Secondary formulation vessel |
| fillfin/FillLine01 | FillLine01 | Vial fill-and-finish line 1 |
| fillfin/FillLine02 | FillLine02 | Vial fill-and-finish line 2 |
| fillfin/Lyophilizer01 | Lyoph01 | Lyophilizer (freeze-drying unit) |

## 3.4  Utilities

| UNS Path Segment | Unit ID | Description |
| --- | --- | --- |
| utilities/WFI | WFISystem | Water for injection system |
| utilities/CleanSteam | CSTSystem | Clean steam generation |
| utilities/HVAC | HVACZone01 | Cleanroom HVAC monitoring |
| utilities/GasPanel | GasPanel01 | N2 / CO2 / O2 distribution panel |
| utilities/Chiller | Chiller01 | Process chilled water loop |

# 4.  Batch Namespace  (ISA-88)

The batch branch contextualises all process data against a specific batch identifier and its ISA-88 procedural elements. This enables complete batch genealogy from a single UNS path without requiring joins or post-processing.

## 4.1  Batch Topic Structure

EnterpriseC/SiteA/batch/{Area}/{Unit}/{BatchID}/{Phase}/{Tag}

Example — temperature reading during the Growth phase of batch B2024-250-001 on Unit 250:

EnterpriseC/SiteA/batch/bioprocess/Unit250/B2024-250-001/Growth/temperature

## 4.2  ISA-88 Procedural Model

| ISA-88 Element | UNS Segment | Description |
| --- | --- | --- |
| Procedure | (BatchID) | Top-level recipe execution — maps to batch ID |
| Unit Procedure | (Unit) | Execution on a specific equipment unit |
| Operation | {Phase} | Major step within the unit procedure |
| Phase | {SubPhase} | Atomic control step (e.g., Ramp, Hold, Transfer) |

## 4.3  Defined Phases — Bioreactor (Unit 250)

| ISA-88 Phase | Typical Duration | Key Monitored Tags |
| --- | --- | --- |
| Charge | 2–4 h | volume, temperature, agitation |
| Sterilize | 1–2 h | temperature, pressure, hold_time |
| Inoculation | 0.5 h | volume_added, transfer_rate, temperature |
| Growth | 48–72 h | temperature, ph, dissolved_o2, agitation, co2_pct, cell_density |
| FedBatch | 96–168 h | temperature, ph, dissolved_o2, feed_rate, base_rate, viability |
| Harvest | 4–8 h | volume_harvested, transfer_rate, temperature |
| CIP | 2–3 h | caustic_concentration, rinse_conductivity, temperature |

## 4.4  Defined Phases — TFF

| ISA-88 Phase | Typical Duration | Key Monitored Tags |
| --- | --- | --- |
| Equilibration | 0.5–1 h | feed_pressure, conductivity, flux |
| Concentration | 2–4 h | feed_pressure, retentate_pressure, tmp, flux |
| Diafiltration | 3–6 h | conductivity, ph, permeate_flow, dv_count |
| Recovery | 0.5 h | volume_recovered, pressure |
| CIP | 1–2 h | caustic_flow, rinse_conductivity, temperature |

# 5.  Full UNS Path Examples

| Full UNS Path | Branch | Source Tag |
| --- | --- | --- |
| EnterpriseC/SiteA/process/bioprocess/Unit250/temperature | process | bioreactor_data.temperature_pv |
| EnterpriseC/SiteA/process/bioprocess/Unit250/ph | process | bioreactor_data.ph_pv |
| EnterpriseC/SiteA/process/bioprocess/Unit250/equipment_state | process | bioreactor_data.equipment_state |
| EnterpriseC/SiteA/batch/bioprocess/Unit250/B2024-250-001/Growth/temperature | batch | bioreactor_data.temperature_pv |
| EnterpriseC/SiteA/batch/bioprocess/Unit250/B2024-250-001/FedBatch/feed_rate | batch | bioreactor_data.feed_rate_pv |
| EnterpriseC/SiteA/process/tff/TFF260/tmp | process | tff_data.tmp_calculated |
| EnterpriseC/SiteA/batch/tff/TFF260/B2024-TFF-001/Concentration/flux | batch | tff_data.flux_pv |
| EnterpriseC/SiteA/process/utilities/WFI/conductivity | process | wfi_data.conductivity_pv |
| EnterpriseC/SiteA/events/bioprocess/Unit250/alarm | events | alarm_log.event_text |

# 6.  AnyLog Historian Mapping

The AnyLog distributed historian stores data in a flat table structure. The UNS path is reconstructed at query time by combining the database name, table name, and tag column. The REST proxy injects the appropriate headers for each query type.

## 6.1  Database / Table Convention

| UNS Area | AnyLog Database | Primary Tables |
| --- | --- | --- |
| bioprocess | enterprise_c | bioreactor_data, seed_data, media_data |
| tff | enterprise_c | tff_data |
| formulation | enterprise_c | formulation_data |
| fillfin | enterprise_c | fillfin_data, lyophilizer_data |
| utilities | enterprise_c | wfi_data, hvac_data, gas_data, chiller_data |
| events | enterprise_c | alarm_log, event_log, audit_log |

## 6.2  REST Proxy Query Examples

Process query (via REST proxy) — no destination header override needed, network routing is automatic:

POST /api/query

{

"dbms": "enterprise_c",

"sql":  "SELECT timestamp, temperature_pv, ph_pv, dissolved_o2

FROM bioreactor_data

WHERE unit_id = 'Unit250'

AND timestamp >= NOW() - 1 hour

ORDER BY timestamp DESC"

}

Batch-contextualised increment query:

POST /api/query/increment

{

"dbms":           "enterprise_c",

"table":          "bioreactor_data",

"timeColumn":     "timestamp",

"startTime":      "2024-01-15 00:00:00",

"endTime":        "2024-01-17 00:00:00",

"timeUnit":       "hour",

"intervalLength": 1,

"projections":    ["avg(temperature_pv)", "avg(ph_pv)", "avg(dissolved_o2)"]

}

# 7.  Naming Conventions

## 7.1  Rules

- All path segments use lowercase_snake_case

- Abbreviations are standardised and listed in Section 7.2

- Unit IDs preserve mixed case where they match physical asset tags (Unit250)

- Batch IDs follow pattern: B{YYYY}-{UnitShort}-{SeqNo}  (e.g., B2024-250-001)

- Numeric suffixes are zero-padded to 3 digits (Unit250, TFF260, FillLine01)

- Paths must not exceed 256 characters

- Reserved words: process, batch, events, meta — do not use as unit or tag names

## 7.2  Standardised Abbreviations

| Abbreviation | Full Term | Usage Context |
| --- | --- | --- |
| pv | Process Variable | Live sensor reading tag suffix |
| sp | Setpoint | Control setpoint tag suffix |
| do | Dissolved Oxygen | Bioreactor dissolved oxygen sensor |
| tmp | Trans-Membrane Press | TFF derived pressure tag |
| tff | Tangential Flow Filt | Process area prefix |
| wfi | Water For Injection | Utility area |
| cip | Clean-In-Place | ISA-88 phase name |
| cst | Clean Steam | Utility area |
| fillfin | Fill and Finish | Process area prefix |

# 8.  Open Items  &  Validation

The following items must be validated against live AnyLog data before this specification is finalised:

| # | Item | Owner | Status |
| --- | --- | --- | --- |
| 1 | Confirm actual AnyLog database name(s) via REST proxy /api/databases | AnyLog / Mark | Open |
| 2 | Enumerate all tables and confirm mapping to UNS areas | AnyLog / Mark | Open |
| 3 | Validate column names match tag names in this spec | AnyLog / Mark | Open |
| 4 | Confirm batch_id column exists in bioreactor_data and tff_data | Mark | Open |
| 5 | Define formulation / fillfin tag namespaces (data not yet observed) | Process Eng. | Open |
| 6 | Confirm timestamp duplication issue is resolved before batch path design finalised | Mark | In Progress |